| Information Delivery Specifications IDS |  |
| --- | --- |
| MiniGIM IDS Niveau 3 Input GREX |  |
| Version: | 0.8 |
| Date: | 01/06/2026 |
| Author Email : | vragen@minigim.nl |
| Milestone: | GREX |
| Purpose: | Niveau 3 |
| Copyright: miniGIM consortium |  |

Created with usBIM.IDSeditor by ACCA software S.p.A.

Specifications

IfcSite

IFC versions: IFC4 IFC4X3_ADD2

The model MUST contain entities that have

-	IFC class IFCSITE

that MEET the following requirements

- 	...

ProjectedCRS

IFC versions: IFC4 IFC4X3_ADD2

The model MUST contain entities that have

-	IFC class IFCPROJECTEDCRS

that MEET the following requirements

-	MUST HAVE attribute VerticalDatum = EPSG:28992 (Instructions: Doel coördinatenstelsel waarin het BIM-model geplaatst wordt. )

Map conversion

IFC versions: IFC4 IFC4X3_ADD2

The model MUST contain entities that have

-	IFC class IFCMAPCONVERSION

that MEET the following requirements

-	MUST HAVE attribute Eastings

-	MUST HAVE attribute Northings

-	MUST HAVE attribute Scale

Elementen

IFC versions: IFC4 IFC4X3_ADD2

The model MUST contain entities that have

-	IFC class IFCBUILDINGELEMENTPROXY

that MEET the following requirements

-	MUST HAVE property NetSurfaceArea of PSet Qto_BuildingElementProxyQuantities

-	MUST HAVE property NetVolume of PSet Qto_BuildingElementProxyQuantities

-	MUST HAVE property Soort of PSet ePset_miniGIM (IFCLABEL) with value = Tuin (or:  = Water;  = Parkeren;  = Parkeren prive;  = Weg;  = Voetpad;  = Spoorbaan;  = Plantvak;  = Berm;  = Wadi;  = Park;  = Speelvoorziening;  = Kolk;  = Boom;  = Straatmeubulair;  = Verlichting;  = Kunstwerk;  = Brug) (Instructions: Deze termen moeten worden uitgebreid en gestandardiseerd. )

-	MUST HAVE any material (Instructions: Voeg een materiaal toe voor de berekeningen.)

-	MUST HAVE property Reference of PSet Pset_BuildingElementProxyCommon (IFCIDENTIFIER) with value = Type I (or:  = Type II;  = Type III;  = n.a.)